Ref:

09/04/2015

Japanese Knotweed

Fallopia japonica

Method Statement for removal

Llandaff Campus, 200 Western Ave, Cardiff CF5 2YB

1.0 Introduction

1.1 Method Statement for the eradication of Japanese Knotweed from the land at the rear of Llandaff Campus, 200 Western Ave, Cardiff CF5 2YB

1.2 This Method Statement outlines the course of action that needs to be taken to eradicate this invasive weed in preparation for the installation of an additional grass surface sports pitch. It is proposed that Gavin Jones staff that have the expertise in the eradication of this weed and who have the necessary experience and qualifications to apply herbicides and who will abide by the details contained within this Method Statement. This Method Statement should be read in conjunction with the Environment Agency Code of Practice regarding Japanese Knotweed.

2.0 The Site

2.1 The site is to the rear of Llandaff Campus, 200 Western Ave, Cardiff CF5 2YB and accessed across an overflow car park and existing sports field which are surrounded by mature woodland and existing Japanese Knotweed which is currently being managed.

2.2 The proposed development will require the construction of an additional football pitch. Some material will be exported from site. New top soil will need to be brought in for areas to be levelled.

2.3 During a recent survey an area of Japanese Knotweed with at least ten shoots showing was discovered within the area of the proposed sports pitch.

3.0 Method Statement Objectives

3.1 The first objective is to contain the existing Japanese Knotweed on site and prevent further spread across the site.

3.2 An agreed control method and programme of works has been in place for approximately two years with regards the eradication of the existing Japanese Knotweed on site to eliminate its spread.

3.3 To monitor the site after development to ensure the site is clear of Japanese Knotweed.

4.0 Methods of Removal

4.1 The method most favoured for a development site is to excavate the area where the Japanese Knotweed is growing and stockpile it elsewhere on site. The stockpiled soil can then be treated with either a contact or residual herbicide. The treatment is quicker as moving the soil to stockpile has the same effect as in that disturbance of the soil will stimulate growth which allows treatment to be carried out more quickly thus reducing the risk of further contamination.

4.2 Others methods such as sifting the soil and taking out the rhizomes would in this instance not be effective as the rhizomes would only be in their infancy.

4.3 Japanese Knotweed and the soil it is growing in can in some instances be buried on site which is a cheaper method of control. However burial on site in this case is not an option due to the soil conditions and the space available on site.

5.0 Herbicides

5.1 It is essential that a competent and qualified person carries out the herbicide treatment. Gavin Jones employees must have the appropriate National Proficiency Tests Council (NPTC) certification. They must carefully follow the instructions on the herbicide label and wear correct PPE.

5.2 Glyphosate contact herbicide, applied April - October, late season preferable.

6.0 Summary

6.1 Japanese Knotweed was first noticed on the site in May 2013 while an ecology assessment was being carried out of the site.

6.3 During the summer of 2013/2014 the existing Japanese Knotweed has been treated and will continue to be managed until total eradication has been achieved.

7.0 Conclusion

7.1 Because the area of infestation is fairly small and the plants as yet have not become too well established it would be possible to eradicate the Japanese Knotweed by digging out the area stockpiling it on site and treat the stockpile in the spring/summer before removal to a licenced tip.

8.0 Method Statement for removal of Japanese Knotweed on land to the rear Llandaff Campus

8.1 The management of the Japanese Knotweed on site shall be overseen by the site Supervisor/ Foreman.

8.2 All information concerning the identification of Japanese Knotweed to be displayed in the site Supervisors/Foremen’s office and mess room facilities.

8.3 The area of Japanese Knotweed should be excavated to a depth of 600mm and stockpiled on an agreed area of the site. This excavation to be carried out as soon as possible.

8.4 The stockpiled area should be clearly signed and stored on a root barrier membrane to avoid further contamination. During the summer the stockpile should be inspected for any signs of growth of Japanese Knotweed and should be treated with either a contact or residual herbicide. After treatment the herbicide should be allowed to take effect and the heap should be turned to encourage further regrowth and then treated again this should be carried out for a complete season.

8.5 The site throughout the development work and for the following year should be inspected for any other signs of Japanese Knotweed. All inspections should be recorded and made available for viewing.

Dylan Lewis

Area Manager

Gavin Jones Ltd

M:07968 993 801

F:01932 828 411

www | twitter | YouTube | facebook